Miltenyi Whole Skin Dissociation Protocol – Adapted for Mouse

Before starting:

- Note that the concentration of enzymes are used as follows (same across all kits):

  - For every 1mL of Buffer (different kits use different total buffer volume):

    - 25uL of Enzyme P

    - 5uL of Enzyme A

    - 100uL of Enzyme D

  - Human epidermis dissociation kit uses 1mL buffer (up to 5 punch biopsies)

  - Mouse epidermis dissociation kit uses 4mL buffer (up to 1.5g tissue)

  - Human whole skin kit uses 500uL buffer (up to 3 punch biopsies)

  - Mouse whole skin kit – if we want to use 500ul buffer, than use 1/8th of the tissue from mouse epidermis kit?  (= ~0.2g).

- Whole Skin Dissociation Protocol

  - Wash skin tissue sample in appropriate buffer or cell culture medium

  - Carefully scrape off the subcutaneous fat using a scalpel.  Store samples in appropriate buffer or cell culture medium until needed.

  - Transfer 435uL Buffer L and 12uL Enzyme P into the gentleMACS C Tube and mix carefully.

    - Note that if wanting to preserve CD4/8 epitopes, do NOT use Enzyme P.

  - Add 50uL of Enzyme D and 2.5uL of Enzyme A into the C tube and mix carefully (keep buffer at the bottom of the tube).

    - Note that Enzyme A and D can be pre-mixed before adding into the C tube.  Do not premix Enzyme P with Enzyme A or Enzyme D.

  - Transfer one sample of skin tissue (4mm human) into the C tube containing the enzyme mix and tightly close it

    - Close C tube tightly beyond the first resistance.

    - Up to 3 samples can be processed per C Tube is an overnight incubation is chosen in the next step.

  - Incubate sample in a water bath at 37C for 3 hours or overnight

    - Longer incubation time increases yield.

  - After incubation dilute the sample by adding 0.5mL of cold cell culture medium

  - Tightly close C Tube and attach it upside down into the sleeve of the gentleMACS Dissociator.

    - Close C tube tightly beyond the first resistance.

    - It has to be ensured that the sample material is located in the area of the rotor/slator.

  - Run the gentleMACS Program h_skin_01

    - Note that the ECM is not completely dissociated after the dissociation step but DO NOT repeat the step as it reduces yield.

  - After termination of the program, detach C Tube from the gentleMACS Dissociator.

  - Perform short centrifugation step to collect the sample material at the tube bottom.

  - Apply cell suspension to ta Pre-Separation Filter, 70um, placed on a 15mL tube

    - Dissociated tissue can be removed from the closed C Tube by pipetting through the septum-sealed opening in the center of the cap of the C Tube.  Use ART 1000 REACH 1000uL pipette tips.

  - Wash the filter with 4mL of cold cell culture medium

  - Discard the filter and centrifuge cell suspension at 300g for 10 minutes at 4C.  Aspirate supernatant completely.

  - Resuspend cells with medium or an appropriate buffer to the required volume for further applications.

  - Process cells immediately.