# Planning the assay:

Make sure your target cells are healthy and that you have enough of them. If the targets are K562 or LCL, avoid splitting them too hard close to when you need to chromate them.  A 1:2 split about three days before chromating should suffice. Since the number of targets/well will be 3x103, you’ll be testing 4 effector:target ratios in sextuplicate and you’ll need targets for the Min/Max, you’ll need at least 2.1 x 105 target cells.

For the Day 20 T cell line to be tested, use 30:1, 10:1, 3:1 and 1:1 ratios. That means 30 effectors to 1 target, 10 effectors to 1 target, etc. See Assay Map (pg. 4) for visual aid. Since the number of targets/well will be 3x103 and all there are 2 target lines set up in sextuplicate, you’ll 2.244 x 106 effector cells total.

# Preparing targets:

You’ll need the K562 Parental and K562-tCD19 cell lines to use as targets.

Count the target cell lines. Use the attached worksheet to record counts and calculations.

Aliquot the appropriate number of target cells into clean 14 mL round-bottom tubes, bring up to 10 mL with fresh LCL medium, and centrifuge at 1500 rpm for 5 minutes at 4ºC.

Resuspend target cells in 1 mL fresh LCL.

Chromate (a.k.a. radiolabel with 51Cr) the K562 targets for 2 hours. Try to use the freshest 51Cr that the lab contains. For each 1 ml of targets to be labeled, use 50 l of 51Cr.  If the 51Cr is over 2 weeks old, use 100 l per 1 ml.

# Preparing effectors during the 51Cr incubation:

See Sample Assay Map (pg. 4) for assistance.

See the worksheet for details on the number of effector cells needed for each effector:target (E:T) ratio.

Count the effector cell line and determine what volume of cells is necessary for each E:T ratio. Record counts and calculations on the attached worksheet.

Place the calculated numbers of cells each in their own 5mL FACS tube, bring up to ~4 mL with fresh LCL medium, and centrifuge at 1500 rpm for 5 minutes at 4ºC.

Resuspend effectors in 1.7 mL LCL medium and transfer each one to its own well in a labeled 12-well plate. From highest to lowest E:T ratio, this produces concentrations of 9x105 cells/mL, 3x105 cells/mL, 9x104 cells/mL, or 3x104 cells/mL.

Using 3 tips on a multi-channel pipettor, aliquot 100 uL effectors for the 30:1 E:T into all the appropriate wells of a clean 96-well plate, as indicated on the Sample Assay Map (p.4).

Using a new set of tips for each effector ratio, repeat the above step for the remaining effectors.

When finished with the effectors, aliquot 3 mL of LCL medium into two clean consecutive wells of the 12-well plate.

Using 6 fresh tips on a multi-channel pipettor, aliquot 100 uL of the LCL Medium into each of the 24 Min wells of the 96-well plate, as indicated in the Sample Assay Map (pg. 4).

Place the plate in the incubator until the target cells are ready.

# Washing targets and setting up assay:

After two hours have elapsed, wash targets with 6 ml of RPMI-HEPES and centrifuge at 1400 rpm for 7 minutes at 4ºC.

Gently pour off the supernatant into a half-filled sand bottle in the hood.

Back on the bench, run each tube across the top of the rack 2-3 times to agitate the cells back into suspension.

Wash cells again with 7 ml RPMI-HEPES and centrifuge at 1400 rpm for 7 minutes at 4ºC.

Gently pour off the supernatant into a half-filled sand bottle in the hood.

Perform one more wash with 7 ml RPMI-HEPES and centrifuge at 1400 rpm for 7 minutes at 4ºC.

Gently pour off the supernatant into a half-filled sand bottle in the hood.

Back on the bench, resuspend each cell pellet in 7.0 mL of LCL medium (final concentration: 3x104 cells/mL per target). For each target, aliquot cells evenly between two clean, consecutive wells of a labeled 12-well plate (= 3.5mL per well).

Using 6 tips on a multi-channel pipettor, aliquot 100uL of the chromated K562-Parental line into all of the appropriate wells of the 96-well plate containing the targets, as indicated on the Sample Assay Map (pg. 4).

Repeat the above step with the chromated K562-tCD19 line.

When finished aliquotting the targets, aliquot 3 mL of the Nonidet P-40/Trypan Blue solution into two consecutive, clean wells of the 12-well plate.

Using 6 fresh tips on a multi-channel pipettor, aliquot 100 uL of the Nonidet P-40/Trypan Blue solution into the 24 Max wells, as indicated on the Sample Assay Map (pg. 4).

Create a balance plate using water and a clean 96-well plate available in the hotlab, balancing it with your plate on the scale.

Centrifuge the plates for 2 minutes at 1200 rpm with brake off (to get the targets and effectors interacting quickly).

Incubate the plate at 37 C in the hotlab incubator for 4 hours.

# Harvesting the assay:

Carefully transfer the plate from the incubator to the bench. If the plate gets jostled or the cells resuspended accidentally, centrifuge the plates for 2 minutes at 1200 rpm with brake off to ensure that the cells are pelleted at the bottom of the wells.

Harvest the CRA by removing 30 l of culture supernatant from the top of each well into its corresponding well on a LUMA plate (which has been properly labeled with initials and date), making sure the orientation of the harvested supernatant matches that of the CRA.  Be careful not to disturb the plate at any time and/or take up any of the cells at the bottom of the wells.

Dry the plates overnight or bake them in the oven if data is needed sooner. When baking the plates, be careful to not let the oven go above 50 C because hotter temperatures can melt the plates. Also place the plates on the second or third shelf to avoid putting them too near the heating element.

***It is very important to dry the plates completely before obtaining a reading on the Topcount because the scintillator in the LUMA Plate can only detect radioactivity on dried samples.**

Obtaining data on the Top Count:

Place a clear Topseal cover over the top of the plate and wipe with antistatic wipes before placing in the Topcount.  Excess static on the covers can cause spurious results.

Carefully follow the instructions to the right of the Topcount.  Select the “51CR Master” protocol.  DO NOT MODIFY any protocols because they were designed to work optimally for the protocol for which they were named.

The data will be printed when the Topcount has finished reading all the plates.  The data can also be saved to a disk and calculated with Excel (recommended).

Notes on hot lab usage and etiquette:

- Please follow all hot lab safety requirements.  Always wear your lab coat and badge when working with radiation.

- Radioactive tape with the date, initials of user, and isotope (51Cr) must be placed on the targets after labeling AND on the plates during incubation.

- After using the benches for setting up/harvesting assays, be sure to empty the pipet bins if they are more than ¾ full.

- Survey all areas where radioactivity has been used: after labeling targets in the hood and on the bench where the assay was set up and harvested.

- Discard assay plates within 2 days of the assay date and make sure to deface the radioactive tape.

- If large radioactive waste disposal box is full, please close and label according to the instructions given in the hot lab orientation.

- If you see that a supply is low or if you are using an unusually large amount of supplies, please let the lab manager know so that replacements can be ordered.

Nonidet P-40/Trypan Blue Solution

5 mL Nonidet P-40 (Calbiochem?)

5 mL Trypan Blue (Sigma T8154)

500 mL Water

## Sample Assay Map

This is a 96-well plate set up for the CRA described above.

K562-Par                K562-tCD19

## 30:1

## 10:1

3:1

1:1

K562-Par Min/Max

K562-Par Min/Max

K562-tCD19 Min/Max

## K562-tCD19 Min/Max

Min Wells                   Max Wells