BD Multiplex Cytokine Bead Array for mouse

Reagent

- Standard stock: 25,000 pg/ml (stored at -80C, 3A4). This stock standards are made from individual flex set cytokines combined to give a single 5x stock. Note IL-4 is stored separately.

- Diluent is in big BD box in the Deli Fridge

* Standard stock contains IL-1b (beads location; E5), IL-3 (A8), IL-5 (A6), IL-6 (B4), IL-9 (B5), IL-10 (C4), IL-12p70 (D7), IL-13 (B8), IL-17A (C5), IL-17F (D6), IFNg (A4), TNFa (C8), MCP-1 (B7), GM-CSF (B9), G-CSF (D4), IL-2 (A5).

* If IL-4 (A7) is needed, you should add IL-4 standard to standard stock

* IL-21 (B6) and IL-23 (D7) are reconstituted individually and frozen down at 10,000pg/ml for top standard (stored at -80C, 3A4). You don’t need to dilute them, but IL-23 cannot be multiplexed because of the same bead position to IL-12p70.

- V-bottom 96-well plate

- 1.5ml Eppendorf tube

Calculate the volume of the beads and diluent beforehand!

To collect serum:

- Bleed mice (if terminal bleed, can get ~500uL) using heparinized tubes into eppendorfs.

- Let sit on ice for 30 min.

- Spin on small rotator 8000RPM x 20 minutes, should have enough for 2 plates

Protocol

- Add 40ul (If IL-4 is needed, add 30ul) assay diluent (pink liquid) to the first well and then 20ul to the next 10 wells in the bottom lane of 96-well. (Note this is a total of 11 contorls)

- Add 10ul of 25,000pg/ml standard stock to the first well (If IL-4 is needed, add 10ul IL-4 standard as well). This dilution is the top standard (5000pg/ml)

- Next pipette well to mix and transfer 20ul of the diluted top standard to the next well. And repeat like this to make serial dilution

* You don’t add the dilution to the last well (blank)

1st well: 5000pg/ml-> 2nd well:2500pg/ml->3rd well: 1250pg/ml…..->10th well: 9.7pg/ml->

11th well: blank

- Pick up the required capture beads and detection antibody for each cytokine

- Take required volume of capture beads and detection antibody diluent and add them to 1.5ml Eppendorf tube separately

* The required volume for these should be calculated with excel sheet

- Mix the capture beads (Blue) well (vortex 15sec). Take required volume of beads for each cytokine and add capture beads diluent.

* The required volume should be the same with that of detection beads

- Mix the detection antibody (Red) well (vortex 15sec). Take required volume of beads for each cytokine and add detection antibody diluent.

- Combine beads and antibody in one tube to make master mix

- Thaw serum samples

- Transfer 5ul standard samples and serum to the well

* Standard samples should be located at the top line of 96-well. Add them reversed order

- Add 10ul of master mix to each well and pipette well

- Incubate the plate for 2 hours at room temperature with foil wrap

- After incubation, add 150ul wash buffer or FACS buffer to each well

- Spin the plate for 3-5 minutes at 1000rpm

- Flick the plate to discard supernatant (use quick wrist motion to flick; dab on paper towel)

- Resuspend the pellet in 100ul wash buffer or FACS buffer per well (pipette well). Note don’t need to pipette as symphony can mix it (you can set how much mixing – usually we do 3).

- Run the samples using the most recent Flex set template

*See detail of HST Experiment set up

- After running samples, conduct cleaning using cleaning template

- Turn off the switch of HTS and put back the sheath line and nozzle the way they were

- Analyze the data with BD FCAP v3.0 software

HTS Experiment set up

- Shut down BD FACS Diva software

- Connect the sheath line to HTS

- Exchange the nozzle (short one) and connect the sample line

- Turn on HTS and switch the mode from normal mode to HTS mode

- Make sure turn on Run low, not stand by mode before priming starts, otherwise sucking valve will be broken.

- Click [HTS]->[Prime] to start priming.

- After priming, make a new experiment layout using Flex set template (You can choose the newest template in the experimental template)

- Double click the tab named “96-well V-bottom” to open plate view

* Originally set standard as specimen 1, and samples as specimen 2

- Select well for standard and samples and adjust load setting as below

Plate format: 96 well V bottom, Throughput mode: Standard

Right click to show plate view.

In loader settings window

Sample Flow Rate: 1.5, Sample volume: 80, Mixing volume: 60, Mixing speed: 200

Number of Mixes: 3, Wash volume: 400

- Optional (Add beads-PE master mix to the blank well to see P1 gate is appropriately located and run properly)

- Right click on the worksheet to show up plate control panel

- Select the first well and click run plate tab or select wells for analysis and click run well

Cleaning wells:

- 4 wells top row with FACS clean; 44 wells H20 on next row.  Connect waterline to plate!!

Data Analysis:

- When exporting data, export singlets only!!

Making up standards

Each cytokine comes with two vials containing lyophilised protein as a standard. These are to be reconstituted with other analytes to allow for multiplexing, except for IL-21 and IL-23.

- Open the vials of the lyophilised standard.

- Pool all the spheres into a 15ml Falcon tube (do NOT pool in the glass vial!!)

- Reconstitute the standards with 400µL of Assay Diluent. Allow for reconstituted standard to calibrate for at least 15mins. Do NOT vortex or mix vigorously.

- Store aliquots of 15µL at -70˚C

- If making up the IL-21 or IL-23 standards, reconstitute to 10,000pg/ml and store aliquots of 50µL – remember this aliquot is your top standard.

Software

BCAS

Login pwd; bmt

- Open Create blue tubes (standards) and red tubes (samples)

- Manually gate samples to label (e.g. “A4”, “A5”, etc….)