[Prot 706] Spatial Transcriptomics of Host Tissue and Gut Microbes

Written 9/11/26 by Albert Yeh. Read from software/microprobe (README.md and CLAUDE.md).

## Notes:

- Two platforms: 10x Xenium (primary) and Singular Genomics (secondary).

- The question is microbe-host interaction in severe gastrointestinal GVHD, so the panel carries custom BACTERIAL probes alongside the host panel — that custom panel is what most of the work goes into.

- Code is at software/microprobe. Run-level documentation lives in a separate GitHub repository, acyeh-lab/ST, under Xenium/<RUN>/.

## 1. Probe design:

- Collect per-organism FASTA for the target species and for the off-target background.

- Run the k-mer conservation analysis to find regions shared across strains WITHIN a target species but unique against the background. Notebooks use the microbe_v1 kernel.

- Screen candidates for specificity, then REVIEW THE BLAST HITS BY HAND. This is a standing requirement, not optional — an automated specificity score is not sufficient evidence to put a probe on a panel.

- When weighing a cross-reactivity hit, weigh it against how abundant that organism actually is in the cohort. A hit against an organism that is not there is not a reason to drop a probe.

- Submit the final candidate set to 10x.

## 2. Imaging:

- Xenium output lands in experiments/<run>/raw/<sampleID>/ as transcripts.parquet and morphology.ome.tif.

- Singular output is a SpatialData zarr.

## 3. Re-segmentation — do not use the instrument's segmentation:

- sbatch shell/Proseg_10x.sh /path/to/sample/dir — the directory must contain transcripts.parquet.

- sbatch shell/Proseg_Singular.sh /path/to/A01.zarr — uses --coordinate-scale 0.3125 and excludes the gene gdna.

- ProSeg jobs: 8 CPUs, 32–128 GB, long runs --time=1-00:00:00 on campus-new.

- FILE FORMAT CHANGE: ProSeg v3 writes expected-counts.mtx.gz (matrix-market), where v2 wrote expected-counts.csv.gz. Older notebooks that read the CSV will NOT work on v3 output without switching to a matrix-market read.

## 4. Analysis:

- Load the ProSeg outputs into a Scanpy AnnData.

- Strip control probes immediately — anything matching Neg* or Unassigned*. Every downstream statistic assumes this has been done.

- Set the spatial coordinates from the cell centroids.

- Process the lysate and no-lysate samples of a pair in parallel; the second sample's id is the first plus ten.

- Pin random seeds to 1234.

## 5. Bacterial density against the ISH image:

- The marimo app registers the post-Xenium ISH image to the instrument DAPI using ORB plus RANSAC affine, then reuses that transform for the 16S channel.

- The ISH tif must be flipped along axis 1 and transposed first. Keep that orientation fix.

## Environment:

- Analysis notebooks use the spatial-xenium kernel; probe design uses microbe_v1.

- Do NOT use the older spatial_v1 kernel — it is missing igraph and leidenalg, so clustering fails with an ImportError.

## Not specified in this protocol:

- The wet-lab side of the Xenium run. This document covers the computational path only.

## Source:

/fh/fast/yeh_a/grp/software/microprobe — README.md, CLAUDE.md and the scripts in shell/. The manual BLAST review requirement and the abundance-weighting of cross-reactivity hits are Albert's standing instructions, recorded 2026-09.