[Prot 207] Violet proliferation dye protocol and CFSE

Material used:

Tag-it Violet Proliferation and Cell Tracking Dye (BioLegend) Product #425101 – excitation 395nm, emission 455 (use 421 readout, BV421/PB channel)

CFSE (stock 5mM, 1C2)

Reconstitution in DMSO:

- Spin down lyophilized reagent/product from each vial

- Prepare 5mM stock solution by reconstituting 1 vial of dye in 50uL anhydrous DMSO

- Aliquot and freeze at -80C, can be stored up to 6mo.  Aliquots can be thawed and refrozen up to 2 times.

Violet Dye Staining (in vivo):

- Wash cells in PBS 1x

- Resuspend cells in 50M/mL in PBS

- Target is 5uM working solution – so dilute 1uL of 5mM stock per 1mL cell suspension (PBS or blue dot media NO FCS!).  May be better to put dye first then add cell suspension for better immediate mixing.  Note you want to disperse dye immediately, so can actually use 500uL cell suspense and dilute 1uL of 5mM dye in 500uL solution before combining the two.

- Vortex immediately to disperse dye

- Incubate at 10min at RT

- Wash with 9x the original volume with media + FBS/FCS (red dot media)

- Repeat step 6 two additional times (total 3x wash)

- Resuspend, count with flow

CFSE labeling (in vivo or in vitro – 1C2)

1.  Resuspend cells at 30x10^6/ml in RPMI without FCS.

2.  Stock CFSE 5mM (-70 C, 2B2) dilute 1/10 in RPMI no FCS (10ul CFSE stock + 90 ul media)

3.  Add diluted CFSE at 4ul/1ml (for in vivo) cell suspension.  May use 1ul/1mL for in vitro.

4.  Incubate 10min/37C/CO2 (mix occasionally)

5.  Wash in complete media (with FCS)